NEXCORT IQ NFC Tag Installation Guide NexCort iQ Campus · NexCort iQ Venue Step-by-step guide to programming and attaching NFC tags to safety reporting signs

| 15 minutes | No technical skills needed | Works on any smartphone |
| --- | --- | --- |

## Overview

An NFC (Near Field Communication) tag is a small sticker — about the size of a coin — that contains a tiny chip. When a smartphone is held near it, the phone automatically opens a web page. No camera, no app download, no typing required.

For NexCort iQ, each NFC tag is programmed with a unique reporting URL tied to your agency and location. When someone taps the sign, they go directly to the safety reporting form for that exact location. The report is received instantly by your security team.

| NOTE | NFC tags and QR codes on the same sign point to the same URL. They work identically — NFC is just faster for users with newer phones. |
| --- | --- |

## What You Need

Create the location in the NexCort iQ web app, then program tags with the RC Mobile App. Buy blank NTAG213 stickers (Amazon is fine for tags only).

| Item | Details | Where to Buy |
| --- | --- | --- |
| NTAG213 NFC stickers | Pack of 100 • Round or square • White finish | Amazon — search “NTAG213 NFC stickers” (~$15–20 per 100) |
| Smartphone | iPhone 7 or newer • Any Android with NFC (most phones since 2014) | You already have this |
| RC Mobile App | Writes NFC tags and scans/verifies QR codes · iOS and Android · no NFC Tools or other apps | App Store / Google Play — search NexCort iQ |
| Location created in NexCort iQ | Create the QR/NFC code in the web app. The RC Mobile App writes the URL — do not paste it into a third-party writer. | app.rapidcortex.us → QR & NFC |
| Permanent marker (optional) | For labeling tag backs with location name | Any office supply store |
| Isopropyl alcohol wipes (optional) | For cleaning sign surface before adhering tag | Amazon or pharmacy |

| TIP | NTAG213 is the most compatible NFC chip type. Do not use NTAG215 or NTAG216 — they work but cost more for no benefit in this use case. |
| --- | --- |

| 1 | Get the Reporting URL Log in to NexCort iQ and generate the URL for this sign location |
| --- | --- |

Each physical sign location gets its own unique URL. You generate this in the NexCort iQ admin dashboard.

| 1 | Log in to the NexCort iQ admin dashboard Go to app.rapidcortex.us and sign in with your admin account. |
| --- | --- |
| 2 | Navigate to QR & NFC Codes From the sidebar, select your agency, then click “QR & NFC Codes.” RC platform admins go to NC Admin → Agencies → [Agency] → QR & NFC Codes. |
| 3 | Click “+ New QR / NFC Code” Enter the name and location of this sign (e.g., “McKinley Hall — 3rd Floor”), select the report type, and click Save. |
| 4 | Do not copy the URL into another app The RC Mobile App writes the correct URL when you tap Program NFC Tag. You do not paste anything into NFC Tools or any other writer. |

| WARNING | Create a separate code for each physical sign location. Do not reuse the same URL across multiple locations — you will not be able to tell which location a report came from. |
| --- | --- |

| 2 | Program the NFC Tag Use the NexCort iQ Mobile App to write the URL to the tag — no third-party tools. |
| --- | --- |

This takes about 10 seconds per tag once you know the steps. You only need to do this once per tag.

| 1 | Download the RC Mobile App Search “NexCort iQ” in the App Store (iOS) or Google Play (Android). Sign in with the same account you use on the web. |
| --- | --- |
| 2 | Open QR & NFC Select your agency, then open the location you created in the web app. |
| 3 | Tap Program NFC Tag The app activates NFC and waits for a tag. You do not paste a URL. |
| 4 | Hold an NTAG213 to the back of your phone iPhone: near the top edge. Android: center back. Hold still for about 2 seconds. |
| 5 | Wait for Tag programmed successfully Then tap the tag to confirm the NexCort iQ report form opens. Use Write Another for extra tags at the same location. |
| 6 | Stay in the RC app Do not switch to NFC Tools or any other NFC utility. NexCort iQ writes and verifies the tag. |

| TIP | If the write fails, try moving the tag slowly around the back of your phone until you find the NFC antenna location. On iPhone, it is near the top edge. On Android, it is usually in the center. |
| --- | --- |

| NOTE | If you are programming multiple tags for the same location, you can write the same URL to as many tags as you need without going back to the dashboard. Each tap on a tag will be counted separately. |
| --- | --- |

| 3 | Test the Tag Before Attaching Always test before mounting — you cannot reprogram a tag once it is under a sign |
| --- | --- |

Testing takes 30 seconds and prevents having to remove a mounted tag if something goes wrong.

| 1 | Lock your phone screen, then hold the tag to the back The phone must be awake (screen on). Hold the programmed tag to the NFC location on the back of your phone without opening any app. |
| --- | --- |
| 2 | A notification should appear at the top of your screen Your phone will show a banner or prompt saying the tag was detected. Tap it. |
| 3 | The NexCort iQ report form should open in your browser You should see the safety reporting form for your location. The page header will show the vertical (Campus Security, Venue Security, etc.). |
| 4 | Check the NexCort iQ dashboard After tapping, log in to the admin dashboard and confirm that the NFC Tap Count for this code has increased by 1. |

| CAUTION | If the form does not open: confirm that NFC is enabled on your phone (Settings → NFC), rewrite the tag from the RC Mobile App (Program NFC Tag), and make sure you are holding the tag in the right position. |
| --- | --- |

| 4 | Attach the Tag to the Sign Peel, stick, and you are done |
| --- | --- |

| 1 | Label the tag (optional but recommended) Before peeling, use a permanent marker to write the location name on the white front of the tag (e.g., “MH-3F” for McKinley Hall 3rd Floor). This helps if you ever need to identify a tag that has come loose. |
| --- | --- |
| 2 | Clean the mounting surface (optional) Wipe the back of the sign with an isopropyl alcohol wipe and let it dry for 30 seconds. This ensures a stronger bond. Skip this step if the surface is already clean. |
| 3 | Peel the backing and press firmly onto the back of the sign Center the tag on the back of the sign. Press firmly for 10–15 seconds. The adhesive sets within a few minutes. |
| 4 | Mount the sign Install the sign in its final location. The NFC tag is now live. Anyone with a smartphone can tap the front of the sign to report. |
| 5 | Do a final tap test after mounting Hold your phone to the front of the mounted sign where the tag is on the back. Confirm the form still opens correctly at the installed location. |

Sign Material Compatibility

NFC works differently depending on what the sign is made of.

| Material | NFC Works? | Notes |
| --- | --- | --- |
| Acrylic / Plexiglass | ✅ Yes | Best option. Tag reads clearly through any thickness. |
| PVC / Foam Board | ✅ Yes | Reads through up to 1/2 inch thickness. |
| Plastic | ✅ Yes | Works on any non-metallic plastic. |
| Wood | ✅ Yes | Works through standard sign thickness. |
| Glass | ✅ Yes | Reads through glass. Stick tag to inner surface. |
| Aluminum (face) | ⚠️ Partial | Aluminum blocks NFC. Mount tag on the face (not back) in a corner, or use a mounting block. |
| Solid metal back | ❌ No | Metal completely blocks the signal. Use a non-metallic mounting plate between tag and metal, or attach to a non-metal surface nearby. |

| 5 | Troubleshooting Common issues and how to fix them |
| --- | --- |

| Problem | Solution |
| --- | --- |
| Phone does not detect the tag at all | Check that NFC is enabled: iPhone → Control Center → NFC icon. Android → Settings → Connected Devices → NFC. Move the tag slowly across the back of the phone to find the antenna. |
| Tag detected but wrong page opens | The tag was programmed with the wrong URL. Open the location in the RC Mobile App and tap Program NFC Tag again. You can overwrite an NTAG213 at any time. Do not use NFC Tools. |
| Tag detected but no page opens | Rewrite the tag from the RC Mobile App so it receives a full https:// NexCort iQ URL. Do not use a third-party NFC writer. |
| Tag stopped working after sign was mounted | The tag may be too close to a metal surface. Move it to a non-metallic area of the sign, or use an on-metal NFC tag (sold separately, labeled “anti-metal” or “on-metal NFC”). |
| Tap count not updating in dashboard | The engagement is tracked when the reporting form loads. If the report page is blocked or times out, the count will not increment. Check your internet connection at the sign location. |
| Sign shows correct form but wrong location name | Go to the NexCort iQ dashboard, find this QR code record, and update the Zone Name field. Changes take effect immediately without reprogramming the tag. |
| Tag adhesive came loose | Clean the surface with isopropyl alcohol, let dry fully, and re-stick the tag. For permanent installations, use a small amount of clear super glue around the tag edge. |

| 6 | Quick Reference Card Print and keep with your NFC tag supplies |
| --- | --- |

| Supplies Checklist ☐ NTAG213 NFC sticker tags ☐ Smartphone with NFC ☐ RC Mobile App (not NFC Tools) ☐ RC admin login at app.rapidcortex.us ☐ Permanent marker (optional) ☐ Isopropyl wipes (optional) | Installation Steps 1. Create location: app.rapidcortex.us → QR & NFC 2. Open RC app → location → Program NFC Tag 3. Hold NTAG213 to the phone until success 4. Test tap — confirm form opens 5. Optional: Scan QR Code in the RC app 6. Label tag with location name 7. Clean sign surface 8. Peel and stick to back of sign 9. Mount sign 10. Final tap test after mounting |
| --- | --- |

Need Help? Contact NexCort iQ Support support@rapidcortex.us · www.rapidcortex.us